REACTOR CONTROL ROOM — OPERATIONS HANDBOOK

From first touch to a safe shift

This guide explains every control, every gauge, and every incident in the simulator — from the underlying principle to the concrete action. Aimed at level 2/5 (beginner with some first experience).

# Quick start

If you want to jump straight in:

- Don’t touch anything at first, just observe. The reactor already starts in a safe equilibrium. Watch for a minute before you adjust anything.

- The bar at the top is your early-warning system. If text appears there, read it — it tells you exactly what the problem is right now.

- The red button is your emergency exit. When in doubt: press SCRAM. It costs points, but never the shift.

- Make small adjustments on the controls. The reactor responds sluggishly — turning a lot, quickly, almost always overshoots.

# 1. Goal of the shift

You take over the control room of a pressurized water reactor for one shift. Your job: generate as much energy as possible without endangering the plant. At the end of the shift — or if an incident escalates — you get a score.

The shift length determines how long a round lasts:

| Shift length | Simulated time | Real play time |
| --- | --- | --- |
| Short | 2 hours | ≈ 10 minutes |
| Normal | 8 hours | ≈ 40 minutes |
| Long | 24 hours | ≈ 2 hours |

To get started: Trainee + Short. That lets you see the whole flow in ten minutes without being caught off guard by an incident.

# 2. The four controls

All four controls are sliders from 0–100%. You can touch them at any time, even during an incident.

## Control rods

Regulate reactivity: 0% = fully withdrawn (maximum power), 100% = fully inserted (shut down). Around 33% the reactor is in equilibrium (critical).

## Main coolant pump

Drives the coolant through the primary loop and the core. If it fails or is throttled, heat builds up in the fuel. Normal operation: 100%.

## Feedwater pump

Supplies the steam generator with water for the secondary loop. It cools the primary loop — even after a SCRAM, once the turbine has already stopped. Normal operation: 100%.

## Turbine valve

Lets steam flow to the turbine — that’s what generates your energy and your points. It closes automatically on a SCRAM. Normal operation: 100%.

Rule of thumb: the control rods make power, the two pumps keep it cool, the turbine valve turns it into points.

# 3. Emergency systems

## SCRAM (emergency shutdown)

Immediately drives the control rods fully in. Ends any dangerous situation, but costs points and generates no energy for a while afterward.

## Emergency Core Cooling System (ECCS)

Actively cools the core, even without pump power. Only switch it on during a loss of coolant — it generates no electricity and cools very aggressively.

## Emergency diesel generator

Kicks in during a grid outage and keeps the pumps running at reduced power until the external grid returns.

# 4. Reading the gauges

Six bars show the state of the plant. Each has a red mark — that’s the critical threshold, not the top of the scale.

| Gauge | Normal | Warning at | Critical at | Meaning |
| --- | --- | --- | --- | --- |
| Power | ≈ 100% | 110% | 122% | Above 122% the automatic system triggers a SCRAM. |
| Fuel temp. | ≈ 600 °C | 780 °C | 850 °C | Above 1200 °C, meltdown is imminent. |
| Coolant temp. | ≈ 315 °C | 335 °C | 345 °C | Temperature in the primary loop. |
| Primary pressure | ≈ 155 bar | 172 bar | 195 bar | Above 172 bar the relief valve opens automatically. |
| Coolant level | 100% | < 80% | < 60% | Only drops on a leak (LOCA). |
| Xenon-135 | low | 55 | — | Neutron poison; rises mainly after a power reduction. |

# 5. How criticality works

Picture a scale: on one side is how many neutrons are newly created in the core (more, the further the rods are withdrawn). On the other side is everything that absorbs neutrons — mainly the control rods themselves, but also temperature and a poison called xenon-135. When the scale balances, power stays constant. When it tips, power rises or falls — and it does so faster the longer it stays out of balance.

That’s why a single large move on the rod control is often enough to let power run away. Small corrections, wait, correct again — that’s really the core of the game.

## The xenon trap

After a power reduction or SCRAM, xenon-135 builds up for a few minutes rather than falling — an aftereffect of the preceding operation. During this time you need the rods further withdrawn to hold the same power. That’s normal, not a display error. It fades again on its own after a few minutes.

# 6. Your first shift — step by step

- Choose Trainee + Short and start the shift. The reactor is already running stably at around 70% power. Just watch how the bars move during normal operation first.

- Nudge the control rods by 2–3% and observe. Pull the slider slightly out (toward 0%). Power rises with a noticeable delay. Wait for it to settle before correcting again.

- Move back toward the starting position. Practice the opposite: insert the rods slightly again until power falls back and stabilizes. This builds a feel for the delay.

- Wait for the first incident. The warning bar at the top and the card under “Active incidents” explain what happened and — if needed — which button to press.

- When in doubt: SCRAM. If a gauge makes you uneasy, press the emergency shutdown. That’s not a failure — it’s the right response and costs far less than a meltdown.

- Shift over — read the result. At the end you see energy generated, number of SCRAMs, relief-valve releases, and your score. That’s your baseline for the next round.

# 7. Incident reference

Incidents occur at random; their frequency depends on the difficulty level. Each appears as a card under “Active incidents” with an explanation.

| Incident | What happens | What to do |
| --- | --- | --- |
| Main coolant pump failure | The primary pump fails, cooling drops. | Press “Acknowledge / Reset” on the incident card. |
| Grid outage | The external grid fails, pump power drops. | Switch on the emergency diesel until the grid returns. |
| Control rod bank stuck | Rods temporarily don’t respond to the control. | Wait it out — resolves itself after a short time. |
| Small Loss-of-Coolant Accident (LOCA) | Coolant level drops continuously. | Switch on ECCS and keep it active for several seconds until the leak is isolated. |
| Sensor fault | Gauges show noisy, unreliable readings. | Treat readings with caution, don’t overcorrect in a panic. |
| Reactivity disturbance | Brief noise in reactivity, power fluctuates slightly. | Trim the control rods with small corrections. |

# 8. Score & difficulty

Your score is essentially the energy generated in MWh, minus penalties: every SCRAM costs points, so does every relief-valve release, and sustained high fuel temperature slowly eats into the score. A meltdown ends the shift immediately with a heavy penalty.

| Difficulty | Incident frequency | Control behavior | Rod speed |
| --- | --- | --- | --- |
| Trainee | low | more forgiving | fast |
| Shift Supervisor | normal | normal | normal |
| Incident Expert | high | more sensitive | sluggish |

# 9. Tips for level 2

- Only ever change one control at a time — otherwise, at the next swing, you won’t know which setting caused it.

- Deliberately wait a few seconds after each correction. The reactor responds with a delay — correcting again immediately almost always leads to overshoot.

- A warning is not an emergency. Only a critical (red) value, or losing track yourself, calls for a SCRAM.

- Xenon builds up after every SCRAM — power becomes harder to hold for a while afterward. That’s normal, not a new bug.

- Once “Trainee + Short” feels stable, move up to “Shift Supervisor” — the core mechanics stay the same, only pace and incident frequency increase.

Handbook for the Reactor Control Room simulator. All thresholds and control behavior match the current game version.